Hottop Profile Notes - Sutter

| Temperature | Event | Action |
| --- | --- | --- |
| 167° F | Pre Heat | Indicated temperature reached |
| 280-290 ° F |  | Turn the roaster heating element down to about 70% |
| 300 ° F | Grassy smell to the beans |  |
| 310-320 ° F | At the height of this phase, the beans will be bright green | Turn the roaster heating element down to about 50% Set fan to 25-50% Goal: Hold temperature of 300-325 degrees It is better for temperature to rise slowly rather than to drop |
|  | During the next 4 minutes, the beans turn from bright green to tan | Turn up the temperature to 100% Smoke will rise Turn on the fan |
| 350-365 ° F |  | Turn the heating element down to prolong the roast The bean temperature will continue to rise |
| 360 ° F | The display temperature and the bean temperature will closely match each other | Now things begin to happen quickly During first crack, turn down the heating element to 50% You may need to turn up the fan |
| 400 ° F | First crack is about to end | Aim for about 2 minutes silence between end of first crack and start of second crack |

| Temperatures possible by segment |  |  |
| --- | --- | --- |
| Segment | Low Temp | High Temp |
| 1 | 158° | 239° |
| 2 | 212° | 297° |
| 3 | 257° | 333° |
| 4 | 284° | 351° |
| 5 | 302° | 369° |
| 6 | 329° | 396° |
| 7 | 347° | 414° |
| 8 | 374° | 424° |

First safety point - 374° F
Second safety point - 395° F
The roaster automatically ejects beans if the temperature reaches 428° F.

No segment can be longer than 3 minutes
Program length must be 10:00-22:00 minutes
A program can run for :00 seconds
Pressing + or – adds or subtracts time in 30 second intervals up to 3 minutes total change (six presses of the button)

DYNAMIC ROASTING CONTROL

While a roast is taking place, the only parameter of a Program that can be modified is the programmed temperature for the active Segment. The "+" and "-" buttons can be used to change the active Segment's set temperature, effectively allowing the user to turn the heating element on or off by temporarily adjusting the segment's temperature goal, but only within that Segment's allowed temperature range and programmed time limit.

The ability to modify temperature in this way is available in all segments until the last 40 seconds of a Program. At that time the Roaster goes into its end mode. The display shows "PLUS TIME" and the function of the "+" and "-" buttons now only allows the user to add plus time, 30 seconds per each press of the "+" until the maximum of an additional 3:00 is added.

The programs above might work well with methods of extraction that are not demanding, but when roasting espresso they would not be the preferred method. In order to learn more about the functioning of this roaster I created the following Program, a modification of the last one shown above:

Eliminate a program segment by setting its time for :01.

A program must have between 4 and 8 segments.

A program must last between 10:00 and 22:00 minutes.

A City Plus roast level means that the roast was stopped at the beginning of second crack.

| Roast Color | Stage | Name of Style | Appearance of Bean |
| --- | --- | --- | --- |
| Very light brown | End of drying | --- | Dry |
| light brown | Start of First Crack | Cinnamon | Dry |
| Light-medium brown | First Crack | Light | Dry |
| Medium brown | End of First Crack | City | Dry |
| Medium-dark brown | Start of Second Crack | Full City (or City Plus) | Dry/Shiny |
| Dark brown | Slow Second Crack | Viennese | Spots of oil |
| Very dark brown | Fast Second Crack | French | Very shiny/oily |
| Extremely dark brown | Fast Second Crack | Spanish | Wet with oil |

Central American, Colombian and African Coffees:  Very acidic or bright.  Stop roast just at the end of First Crack/City.

Brazilian, Mexican and other Mild Coffees:  Low acidity; Roast to between First and Second Crack; City Plus

Indonesian:  Like Brazilian, Try roasting lightly.  For espresso, roast to Full City

Sumatran or Sulawesi:  Roast to Full City or darker.  Good for blending